INDUSTRIAL OBD TRACKING DEVICE

Complete Engineering Design Report

Quectel EC200U  |  ESP32  |  CAN Bus  |  GPS/GNSS  |  4G LTE Cloud

Hardware Design  •  KiCad Component List  •  Pin Connections  •  PCB Guidelines  •  Firmware Architecture

# 1. System Overview & Architecture

This document provides a complete, industrial-grade engineering reference for designing an OBD-II (On-Board Diagnostics) vehicle tracking device. The design reads real-time CAN bus data from a vehicle, acquires GPS/GNSS location via the Quectel EC200U module's built-in GNSS receiver, and transmits all telemetry to the cloud using 4G LTE. The ESP32 microcontroller serves as the central processing unit, chosen for its built-in CAN (TWAI) controller, triple UART interfaces, and low power sleep modes.

## 1.1 Design Goals

- Read CAN 2.0B frames from vehicle OBD-II port (ISO 15765-4 / SAE J1979)

- Acquire real-time GPS/GNSS coordinates using Quectel EC200U built-in GNSS

- Transmit data to cloud via 4G LTE (MQTT over TLS) using EC200U

- Operate continuously on vehicle 12V battery (9V–16V input range)

- Industrial temperature range: -40°C to +85°C

- Hardware watchdog for unattended long-term deployment

- OTA (Over-The-Air) firmware update capability

- ESD and transient protection on all external-facing signals

## 1.2 High-Level Architecture

Vehicle OBD-II Port → [CAN Transceiver SN65HVD230] → ESP32 TWAI → [EC200U UART] → 4G LTE → Cloud (MQTT/HTTPS)   GPS Antenna → EC200U Built-in GNSS → NMEA → ESP32 UART → Parse → Cloud   12V Vehicle Power → LM2596 (5V Buck) → AMS1117 (3.3V LDO) → All logic   USB Type-C → CP2102N (USB-UART) → ESP32 UART0 → Flash / Debug

# 2. Complete Hardware Component List with KiCad Names

The table below lists every component required for the complete OBD tracking PCB. KiCad names are provided in the format used in KiCad's standard symbol and footprint libraries (Device, Connector, Regulator_Linear, etc.). Custom or manufacturer-specific footprints are noted where required.

| # | Component | KiCad Full Name | Specification | Qty | Purpose |
| --- | --- | --- | --- | --- | --- |
| 1 | Microcontroller | ESP32-WROOM-32U | Dual-core 240 MHz, 4 MB Flash, WiFi+BT | 1 | Main MCU — CAN, UART, GPIO |
| 2 | 4G LTE + GNSS Module | Quectel_EC200U-CN (Custom Footprint) | Cat-1 LTE, Built-in GNSS, UART I/F | 1 | 4G Internet + GPS |
| 3 | CAN Bus Transceiver | SN65HVD230DR | 3.3V CAN ISO 11898, 1 Mbps, SO-8 | 1 | CAN physical layer interface |
| 4 | Alt CAN Transceiver | MCP2551-I/SN | 5V CAN, fault-tolerant, SOP-8 | 1 | Alternative — 5V systems |
| 5 | OBD-II Connector | CONN_OBD2_16PIN | Female 16-pin OBD-II (J1962 Type A) | 1 | Vehicle interface |
| 6 | Buck Converter (12V→5V) | LM2596S-5.0 | 3A, 5V Fixed, TO-263-5, 150 kHz | 1 | Main 5V power rail |
| 7 | LDO Regulator (5V→3.3V) | AMS1117-3.3 | 1A, SOT-223, Low dropout | 1 | 3.3V for ESP32 + EC200U |
| 8 | LDO Regulator (5V→1.8V) | AMS1117-1.8 | 1A, SOT-223 | 1 | 1.8V for EC200U I/O if needed |
| 9 | USB-UART Bridge | CP2102N-A02-GQFN28 | USB 2.0 Full Speed, QFN-28 | 1 | Firmware flashing + debug |
| 10 | EEPROM | AT24C256C-SSHL-T | 256 Kbit, I2C, SOP-8 | 1 | Config + data logging |
| 11 | RTC Module | DS3231SN# | ±2 ppm, I2C, SOIC-16, Battery backup | 1 | Accurate timekeeping |
| 12 | Accelerometer/IMU | MPU-6050 | 6-axis MEMS, I2C, QFN-24 | 1 | Vibration + motion detect |
| 13 | SIM Card Holder | SIM_CARD_8PIN_SMD | Nano SIM, push-push, SMD | 1 | SIM for EC200U |
| 14 | LTE Main Antenna | CONN_SMA_EDGE | SMA Edge Mount, 50 Ohm | 1 | 4G LTE antenna connector |
| 15 | GPS/GNSS Antenna | CONN_SMA_EDGE | SMA Edge Mount, active patch | 1 | GNSS antenna connector |
| 16 | CAN Term Resistor | R_120R_0805 | 120 Ω, 0805, 1% | 2 | CAN bus termination |
| 17 | Bulk Capacitor | C_1000uF_25V | 1000 µF, 25V, electrolytic | 2 | Input filtering |
| 18 | Decoupling Cap 100nF | C_100nF_0402 | 100 nF, 50V, X7R, 0402 | 20 | Power decoupling |
| 19 | Decoupling Cap 10µF | C_10uF_0805 | 10 µF, 10V, X5R, 0805 | 10 | Bulk decoupling |
| 20 | Schottky Diode (reverse protect) | B5819WS | 40V, 1A, SOD-323 | 1 | Reverse polarity protection |
| 21 | TVS Diode (12V rail) | SMAJ15A | 15V unidirectional, SMB | 2 | OBD line transient protection |
| 22 | TVS Diode (CAN bus) | PESD1CAN | Dual-line CAN TVS, SOT-363 | 1 | CAN bus ESD protection |
| 23 | Ferrite Bead | BLM21PG221SN1D | 220 Ω @ 100 MHz, 0805 | 4 | EMI filtering on power lines |
| 24 | P-MOSFET (power switch) | AO3401A | P-MOS, 30V, 4A, SOT-23 | 1 | Soft power switching |
| 25 | Reset IC | MCP809T-450I/TT | 3.3V supervisory, SOT-23 | 1 | System power-on reset |
| 26 | LED Status (4x) | LED_0805 | Red/Green/Blue/Yellow, 0805 | 4 | Status indicators |
| 27 | LED Resistors | R_330R_0402 | 330 Ω, 0402 | 4 | Current limiting for LEDs |
| 28 | Pull-up Resistors | R_4k7_0402 | 4.7 kΩ, 0402 | 6 | I2C, UART pull-ups |
| 29 | Filter Inductor | LM2596_INDUCTOR_100uH | 100 µH, 3A, SMD Power | 1 | LM2596 buck inductor |
| 30 | Catch Diode | SS34 | 40V, 3A, Schottky, SMB | 1 | LM2596 freewheeling diode |
| 31 | Debug Header | CONN_01X04_2.54MM | 4-pin 2.54mm UART debug | 1 | UART debug access |
| 32 | JTAG/SWD Header | CONN_01X05_2.54MM | 5-pin 2.54mm | 1 | JTAG programming |
| 33 | Watchdog IC | TPL5110DDCT | Nano-power Timer WDG, SOT-23-6 | 1 | Hardware watchdog reset |
| 34 | I/O Expander (optional) | PCF8574T/3,518 | 8-bit I2C I/O, SOP-16 | 1 | Extra GPIO expansion |
| 35 | Crystal Oscillator | Crystal_32MHz_SMD | 32 MHz, ±10 ppm, 3225 SMD | 1 | ESP32 external crystal |

KiCad Library Paths: Device:*, Connector_PinHeader_2.54mm:*, Regulator_Linear:AMS1117-3.3, Regulator_Switching:LM2596S-5.0, Interface_CAN_LIN:SN65HVD230x, MCU_Module:ESP32-WROOM-32. EC200U requires a custom KiCad symbol and footprint — download from Quectel's official hardware design guide or SnapEDA.

# 3. ESP32 Suitability Analysis for OBD Tracking

## 3.1 Verdict: YES — ESP32 Works Excellently

The ESP32 is a very capable choice for this OBD tracking application. It has a built-in TWAI (Two-Wire Automotive Interface) controller which is fully compatible with CAN 2.0B / ISO 11898-1. Combined with three hardware UART interfaces (one for EC200U 4G, one for GNSS, one for USB debug), the ESP32 handles all communication requirements without any additional CAN controller IC. The following table details the complete capability assessment:

| Parameter | ESP32 Value | Requirement | Status |
| --- | --- | --- | --- |
| CPU Speed | 240 MHz Dual-core | ≥ 80 MHz | ✓ PASS |
| RAM | 520 KB SRAM | ≥ 256 KB for buffers | ✓ PASS |
| Flash | 4 MB (WROOM) | ≥ 2 MB firmware | ✓ PASS |
| UART Count | 3 UARTs | 3 needed (CAN driver, EC200U, USB) | ✓ PASS |
| CAN Support | TWAI Controller (built-in) | CAN 2.0B required | ✓ PASS |
| I2C | 2 × I2C | 1 for sensors | ✓ PASS |
| Operating Voltage | 3.3V | 3.3V system | ✓ PASS |
| Operating Temp | -40°C to +85°C | -40°C to +85°C (automotive) | ✓ PASS |
| Deep Sleep Current | 10 µA | Low power for parking mode | ✓ PASS |
| WiFi | 802.11 b/g/n | Optional local config | ✓ BONUS |
| Bluetooth | BT 4.2 + BLE | OBD config via app | ✓ BONUS |
| Industrial Grade Alt | ESP32-S3 / STM32F4 | Higher reliability | ⚠ CONSIDER |
| Watchdog | Hardware TWDT | Required for automotive | ✓ PASS |

## 3.2 Key ESP32 Advantages for This Design

- Built-in TWAI/CAN Controller: No external CAN controller IC needed — only the SN65HVD230 transceiver (physical layer) is required. Supports CAN 2.0B up to 1 Mbps.

- 3 Hardware UARTs: UART0 (USB Debug/Flash), UART1 (EC200U 4G LTE), UART2 (EC200U GNSS) — all three used simultaneously with no software overhead.

- Extremely Low Power Deep Sleep: 10 µA sleep current — perfect for parking mode (engine off) to prevent draining vehicle battery.

- Dual Core: Core 0 handles CAN data parsing and logging while Core 1 manages EC200U/cloud communication without blocking.

- Built-in WiFi: Enables local OTA updates via hotspot without needing 4G during development.

- Built-in BLE: Enables configuration via mobile app (change APN, MQTT server, etc.) without removing the device.

## 3.3 Limitations & Mitigations

- Not AEC-Q100 Qualified: ESP32 is consumer-grade, not automotive-certified. Mitigation: Use hardware watchdog (TPL5110), external supervisor IC, and conformal coating.

- Flash Wear on Heavy Logging: Frequent flash writes degrade longevity. Mitigation: Use external EEPROM (AT24C256) for config, buffer data in SRAM before writing.

- Industrial Alternative: For high-reliability applications, consider STM32F405 (ARM Cortex-M4, hardware CAN, -40 to +85°C, AEC-Q100 available) or ESP32-S3 for improved RF performance.

# 4. Complete Pin Connections & Signal Map

This section provides a complete wiring reference for the PCB designer and firmware engineer. Every signal between components is defined with GPIO assignment, direction, voltage level, and special notes.

| Pin Group | ESP32 GPIO | Connected To | Signal Name | Notes |
| --- | --- | --- | --- | --- |
| CAN Bus (SN65HVD230) |  |  |  |  |
| CAN TX | GPIO 5 | SN65HVD230 — Pin 1 (TXD) | CAN_TX | 3.3V logic |
| CAN RX | GPIO 4 | SN65HVD230 — Pin 4 (RXD) | CAN_RX | 3.3V logic |
| CAN H | — | Vehicle OBD pin 6 | CANH | 120Ω termination at each end |
| CAN L | — | Vehicle OBD pin 14 | CANL | Twisted pair to connector |
| EC200U — 4G LTE Module |  |  |  |  |
| UART2 TX | GPIO 17 | EC200U — MAIN_RXD (Pin 22) | MCU→EC200U | 115200 baud |
| UART2 RX | GPIO 16 | EC200U — MAIN_TXD (Pin 21) | EC200U→MCU | 115200 baud |
| PWRKEY | GPIO 26 | EC200U — PWRKEY (Pin 17) | MODULE_PWR | Active LOW, >500ms pulse |
| RESET | GPIO 27 | EC200U — RESET_N (Pin 19) | MODULE_RESET | Active LOW |
| STATUS | GPIO 35 | EC200U — STATUS (Pin 20) | MODULE_STATUS | Input — HIGH=on |
| RI | GPIO 34 | EC200U — RI (Pin 25) | RING_IND | Optional wake-up |
| DTR | GPIO 33 | EC200U — DTR (Pin 26) | DTR | Sleep control |
| GPS/GNSS (EC200U Built-in) |  |  |  |  |
| GNSS UART TX | GPIO 13 | EC200U — GNSS_TXD | GNSS_DATA | NMEA sentences @ 9600 |
| GNSS UART RX | GPIO 12 | EC200U — GNSS_RXD | GNSS_CMD | Config commands |
| USB / Debug (CP2102N) |  |  |  |  |
| USB D+ | — | CP2102N — D+ | USB_DP | To USB type-C connector |
| USB D- | — | CP2102N — D- | USB_DM | To USB type-C connector |
| UART0 TX | GPIO 1 | CP2102N — RXD | DEBUG_TX | Programming + log |
| UART0 RX | GPIO 3 | CP2102N — TXD | DEBUG_RX | Programming + log |
| I2C Bus (SDA/SCL) |  |  |  |  |
| I2C SDA | GPIO 21 | EEPROM pin 5 + RTC pin 6 + MPU6050 pin 23 | I2C_SDA | 4.7kΩ pull-up to 3.3V |
| I2C SCL | GPIO 22 | EEPROM pin 6 + RTC pin 5 + MPU6050 pin 24 | I2C_SCL | 4.7kΩ pull-up to 3.3V |
| LEDs & Status |  |  |  |  |
| LED Power | GPIO 2 | Red LED → 330Ω → GND | LED_POWER | ON when system active |
| LED LTE | GPIO 14 | Blue LED → 330Ω → GND | LED_LTE | Blinks when connected |
| LED GPS | GPIO 15 | Green LED → 330Ω → GND | LED_GPS | ON when GPS fixed |
| LED ERR | GPIO 25 | Yellow LED → 330Ω → GND | LED_ERROR | Error/fault indicator |
| Watchdog (TPL5110) |  |  |  |  |
| WDG DONE | GPIO 32 | TPL5110 DONE pin | WDG_FEED | Pulse to reset watchdog |
| WDG RESET OUT | EN pin of ESP32 | TPL5110 RESET OUT | SYS_RESET | Resets ESP32 on timeout |
| Power Rails |  |  |  |  |
| OBD +12V (Pin 16) | — | LM2596 VIN | VIN_12V | 9–16V from vehicle |
| LM2596 OUT (5V) | — | AMS1117-3.3 VIN, EC200U VBAT | VCC_5V | Clean 5V rail |
| AMS1117 OUT (3.3V) | 3.3V VDD | ESP32, SN65HVD230, sensors | VCC_3V3 | Main logic rail |
| OBD GND (Pin 4/5) | GND | All component grounds | PGND/DGND | Star ground topology |

# 5. OBD-II Connector Pin Assignment

The OBD-II 16-pin female connector (J1962 Type A) is the vehicle interface. Only relevant pins are connected — unused pins are left NC (no connect) with optional pull-down resistors for ESD safety.

| OBD-II Pin | Signal | Connected To | Description |
| --- | --- | --- | --- |
| Pin 2 | SAE J1850 Bus+ | Not connected (NC) | Legacy protocol |
| Pin 4 | Chassis Ground | PGND — Power Ground | Vehicle chassis GND |
| Pin 5 | Signal Ground | DGND — Digital Ground | ECU signal GND |
| Pin 6 | CAN High (ISO 15765) | SN65HVD230 CANH | CAN-H — 2.5V–3.5V |
| Pin 7 | K-Line (ISO 9141) | NC or UART RX | Legacy K-Line (optional) |
| Pin 14 | CAN Low (ISO 15765) | SN65HVD230 CANL | CAN-L — 1.5V–2.5V |
| Pin 15 | L-Line (ISO 9141) | NC | Legacy L-Line |
| Pin 16 | Battery Positive (+12V) | LM2596 VIN + TVS Diodes | Main vehicle power 9–16V |

IMPORTANT: Always place a 15V TVS diode (SMAJ15A) between Pin 16 (+12V) and GND, and a reverse-polarity Schottky diode (B5819WS) in series on the +12V line before the LM2596 input. The CAN lines (Pins 6 and 14) must have PESD1CAN TVS for ESD protection per ISO 7637-2.

# 6. Power Supply Design

## 6.1 Power Architecture Overview

The power supply is a two-stage design: a switching buck converter (LM2596) for high-efficiency 12V→5V conversion, followed by linear LDOs for clean 3.3V and 1.8V rails. The EC200U module's VBAT is powered directly from the 5V rail (the module has its own internal regulators). The design must handle the EC200U's peak transmit current of ~2A.

| Stage | Input | Output | Component | Load Current | Efficiency |
| --- | --- | --- | --- | --- | --- |
| Vehicle Input | 9V–16V DC | Protected 12V | TVS + Schottky + Ferrite | 2A max | — |
| Buck 12V→5V | 9–16V | 5V ±2% | LM2596S-5.0 | 3A max | ~85% |
| LDO 5V→3.3V | 5V | 3.3V ±1% | AMS1117-3.3 | 1A max | 66% |
| LDO 5V→1.8V | 5V | 1.8V ±1% | AMS1117-1.8 | 500mA | 36% |
| EC200U VBAT | 5V direct | Internal regulated | Built-in LDO | 2A peak (4G TX) | — |

## 6.2 Critical Power Design Rules

- Place 1000 µF electrolytic capacitor directly at OBD-II +12V input for bulk hold-up and spike suppression.

- EC200U VBAT: Place 100 µF + 10 µF + 100 nF capacitors within 5mm of EC200U VBAT pins. EC200U can draw 2A peak during 4G transmission.

- LM2596 Layout: Keep the LM2596, inductor (100µH), diode (SS34), and output capacitor in a tight loop. Minimize switching node (SW pin) trace area to reduce EMI.

- Separate PGND (power ground) and DGND (digital ground), connected at single star point near the LM2596 GND pin.

- Add a ferrite bead (BLM21PG221SN1D) between 5V and 3.3V power planes at the entry point to prevent switching noise coupling.

# 7. Schematic Block Description (KiCad Implementation)

## 7.1 Schematic Sheet Organization

For clean KiCad schematic organization, create separate hierarchical sheets for each functional block:

- Sheet 1 — Top Level: Shows all sub-sheets as hierarchical blocks with inter-sheet power flags and net labels

- Sheet 2 — Power Supply: LM2596, AMS1117-3.3, AMS1117-1.8, protection diodes, filter caps

- Sheet 3 — ESP32 Module: All ESP32-WROOM-32U pins with net labels, decoupling caps, reset circuit

- Sheet 4 — EC200U Module: All EC200U pins, antenna connectors, SIM holder, UART connections

- Sheet 5 — CAN Interface: SN65HVD230, OBD-II connector, TVS diodes, termination resistors

- Sheet 6 — Peripherals: USB (CP2102N), EEPROM, RTC, IMU, LEDs, watchdog, debug headers

## 7.2 Key Schematic Rules for KiCad

- Use PWR_FLAG symbols on all power nets (+12V, +5V, +3V3, GND) to prevent ERC errors.

- All net names must match between sheets: use identical net labels (CAN_TX, CAN_RX, UART1_TX, etc.) for cross-sheet connections.

- Add test point (TestPoint:TestPoint_Pad_2.0x2.0mm) on all critical signals: CAN_H, CAN_L, +5V, +3V3, UART1_TX, GPS_TX for in-circuit testing.

- EC200U: This requires a custom KiCad symbol. Create a symbol with all 70+ pins organized into functional groups (POWER, UART_MAIN, UART_GNSS, AUDIO, SIM, RF). Download reference schematic from Quectel's EC200U-XX Hardware Design document.

- CAN termination: Place two 120Ω resistors (R_120R_0805) with jumper (0Ω 0402 or solder bridge) in series so termination can be disabled if device is not at bus end.

## 7.3 EC200U Key Pin Connections (Quectel Reference)

EC200U Pin Reference — UART_MAIN: Pins 21(TXD), 22(RXD), 25(RI), 26(DTR). GNSS: Pins for GNSS_TXD/RXD. POWER: VBAT(Pins 1-4), PWRKEY(Pin 17), RESET_N(Pin 19), STATUS(Pin 20). SIM: USIM_VCC, USIM_DATA, USIM_CLK, USIM_RST. RF: MAIN_ANT (LTE), GNSS_ANT. Always refer to Quectel EC200U-XX Series Hardware Design V1.x for exact pin numbers as they may vary by variant (EC200U-CN, EC200U-EU, EC200U-AU).

# 8. PCB Design Guidelines

## 8.1 Layer Stack-up

| Layer | Use | Copper Weight | Notes |
| --- | --- | --- | --- |
| Top (L1) | Component placement + signal traces | 35µm (1 oz) | Primary signal layer |
| GND (L2) | Solid copper ground plane | 35µm | Critical for EMI |
| PWR (L3) | Power distribution plane | 70µm (2 oz) | 5V + 3.3V planes |
| Bottom (L4) | Signal traces + thermal vias | 35µm | High-current traces |
| Via Type | Through-hole vias min 0.3mm drill | — | Blind/buried optional |
| PCB Size | 80mm × 60mm (recommended) | — | Fits standard enclosure |
| Impedance | 50Ω controlled for antenna traces | — | RF path critical |
| Surface Finish | HASL / ENIG recommended | — | ENIG for fine-pitch parts |
| Solder Mask | Green / Black | — | Both sides |
| Conformal Coat | Acrylic AR coating after assembly | — | Moisture protection |

## 8.2 Critical PCB Layout Rules

### Antenna Placement

- LTE antenna connector (SMA): Place at board edge with minimum 15mm keep-out zone from all other components. Route the 50Ω microstrip trace from EC200U RF pin to SMA — trace width ~1.8mm on standard FR4 (calculate with impedance calculator for your actual stack-up).

- GPS antenna connector: Place at opposite edge from LTE antenna. 50Ω controlled impedance trace. Add ground stitching vias along antenna traces every 3mm.

- Do NOT route any signal traces under antenna connectors or RF traces. Place solid ground copper pour.

### CAN Bus Layout

- Route CANH and CANL as a differential pair, 120 mil spacing, equal length. Use 120Ω differential impedance if possible.

- Place PESD1CAN TVS diode as close as possible to the OBD-II connector — within 10mm.

- Place 120Ω termination resistors at the physical end of the CAN trace, near the OBD connector.

### Power Layout

- LM2596 switching loop: Keep the area enclosed by VIN, SW, diode, and output capacitor loop as small as possible to minimize EMI.

- EC200U VBAT decoupling: 100µF + 10µF + 100nF in parallel, placed within 5mm of VBAT pins.

- Pour a solid ground plane on Layer 2 (no cuts or gaps except for via anti-pads).

- Use thermal relief vias under LM2596 exposed pad and ESP32 GND pins.

### Component Placement Priority

- 1st: OBD-II connector (board edge, mechanically fixed)

- 2nd: Antenna connectors (SMA at board edges)

- 3rd: EC200U module (near antennas, away from switching supply)

- 4th: Power supply block (LM2596, LDOs — one corner)

- 5th: ESP32 (center of board for short traces to all peripherals)

- 6th: CAN transceiver (between ESP32 and OBD connector)

- 7th: USB connector (board edge, near CP2102N)

- 8th: All peripheral ICs, decoupling caps, LEDs

# 9. Cloud & Connectivity Architecture

| Layer | Protocol | Platform Options | Data Rate |
| --- | --- | --- | --- |
| Device → Cloud | MQTT over TLS 1.2 | AWS IoT Core / HiveMQ / Mosquitto | ~1–5 KB/s typical |
| Alternate Protocol | HTTPS REST API | Any REST backend / Flask / FastAPI | On demand |
| Time Sync | NTP over EC200U | pool.ntp.org | At boot + hourly |
| OTA Firmware | HTTPS + ESP-IDF OTA | S3 bucket / GitHub Releases | On demand |
| Dashboard | WebSocket / MQTT Sub | Grafana / Node-RED / Custom | Real-time |
| Database | MQTT → InfluxDB / TimescaleDB | Time-series optimized | ~1M records/day |

## 9.1 MQTT Data Payload Format

JSON Payload Example: {   "device_id": "OBD_001",   "timestamp": 1712000000,   "gps": { "lat": 17.3850, "lon": 78.4867, "alt": 536.0, "speed_kmh": 45.2, "hdop": 1.2, "fix": 3 },   "can": { "rpm": 1800, "speed": 45, "coolant_temp": 85, "engine_load": 32, "throttle": 18, "vin": "1HGBH41JXMN109186" },   "power": { "vbat": 12.8, "ign": true },   "imu": { "ax": 0.12, "ay": -0.05, "az": 9.81 },   "signal": { "rssi": -87, "operator": "Airtel", "rat": "LTE" } }

# 10. Firmware Architecture (ESP-IDF / Arduino)

## 10.1 FreeRTOS Task Structure

- Task 1 — CAN_Reader (Core 0, Priority 10): Reads CAN frames using ESP32 TWAI driver. Sends parsed OBD-II PIDs to a FreeRTOS queue at 10 Hz.

- Task 2 — GPS_Parser (Core 0, Priority 8): Reads NMEA sentences from EC200U GNSS UART. Parses GPGGA/GPRMC for location, speed, fix quality.

- Task 3 — Cloud_Publisher (Core 1, Priority 6): Dequeues CAN + GPS data, constructs JSON payload, publishes to MQTT broker. Handles AT command interaction with EC200U.

- Task 4 — Watchdog_Feed (Core 1, Priority 15): Pulses TPL5110 DONE pin every 30 seconds. If any task hangs, this task stops and the hardware watchdog resets the system.

- Task 5 — OTA_Monitor (Core 1, Priority 3): Periodically checks for firmware update via HTTPS. Downloads and applies OTA using esp_https_ota.

- Task 6 — BLE_Config (Core 0, Priority 2): Exposes BLE GATT service for local device configuration (APN, MQTT server, sampling rate).

## 10.2 Power Management States

- ACTIVE Mode: Full operation — CAN sampling 10 Hz, GPS 1 Hz, MQTT publish every 5s. Total current ~450 mA @ 5V.

- IDLE Mode: Engine off detected (no CAN bus activity). GPS + LTE still active, publish rate drops to 60s. Current ~300 mA.

- SLEEP Mode: Parked >10 min. ESP32 light sleep, EC200U PSM (Power Saving Mode) active. GPS disabled. Current ~15 mA. Wake on CAN bus activity.

- DEEP SLEEP: Configurable after extended parking. ESP32 deep sleep 10 µA. EC200U eDRX mode. Wake via RTC timer or ignition detect on GPIO.

# 11. Industrial Design & Reliability Considerations

## 11.1 Enclosure Requirements

- IP54 or higher rated ABS/PC enclosure for under-dash or engine bay deployment.

- Recommended enclosure: Phoenix Contact or Hammond Manufacturing RP series (polycarbonate, IP65).

- OBD-II plug passes through enclosure via strain-relief grommet. Antenna connectors (SMA) mounted on enclosure panels.

- PCB must be secured with M3 standoffs (minimum 4 points) to prevent vibration damage.

## 11.2 EMC / EMI Compliance

- Target compliance: CISPR 25 Class 3 (automotive EMC), ISO 7637-2 (transient immunity).

- Apply conformal coating (Acrylic AR type, MG Chemicals 419C) after assembly and testing.

- Shield EC200U module with metal RF shield can if EMC testing fails on LTE harmonics.

- All external cables must have ferrite cores (Fair-Rite 0431167281) at PCB entry points.

## 11.3 Testing Checklist Before Production

- Power-on test: Verify 5V ± 100 mV and 3.3V ± 50 mV on test points

- CAN bus test: Use Vector CANalyzer or PEAK PCAN-USB to verify frame reception

- GPS cold start fix: Verify NMEA output within 90 seconds (open sky)

- 4G connectivity: Verify AT+CREG? returns registered, PING to server works

- MQTT publish: Verify data arrives at cloud dashboard within 10 seconds

- Temperature soak: -20°C to +70°C, verify operation at extremes

- Vibration test: 5-50 Hz, 0.5G for 2 hours per axis (IEC 60068-2-6)

- OTA update: Verify firmware update completes successfully over 4G

# 12. Quick Reference Summary

| Item | Value |
| --- | --- |
| MCU | ESP32-WROOM-32U (or ESP32-S3 for industrial) |
| 4G + GPS Module | Quectel EC200U-CN / EC200U-EU (variant per region) |
| CAN Transceiver | SN65HVD230DR (3.3V) or MCP2551 (5V systems) |
| Power Input | 9V – 16V DC from OBD-II Pin 16 |
| Power Stages | LM2596S (12→5V) + AMS1117-3.3 (5→3.3V) |
| USB Bridge | CP2102N (USB 2.0 Full Speed) |
| Protocols | CAN 2.0B, MQTT/TLS, NMEA, AT Commands, I2C, UART |
| PCB | 4-Layer, 80×60mm, FR4, ENIG finish |
| KiCad Version | KiCad 8.x (Recommended for best library support) |
| Operating Temp | -40°C to +85°C (conformal coated) |
| Typical Current | ~450 mA @ 5V active ¦ ~15 mA sleep mode |
| Cloud Protocol | MQTT over TLS 1.2 (AWS IoT / HiveMQ / custom broker) |

NEXT STEPS: (1) Download Quectel EC200U Hardware Design Guide + KiCad footprint from Quectel website. (2) Download SN65HVD230 reference schematic from TI. (3) Begin KiCad schematic with hierarchical sheets as described in Section 7. (4) Order evaluation modules: ESP32-DevKitC, EC200U-CN EVB, OBD-II breakout board. (5) Prototype and test CAN → MQTT data pipeline before PCB design.