# Functional Test Procedure Document

## ESP32 + A7672 OBD Vehicle Tracking PCB

# Project Overview

This PCB is an ESP32 + A7672 based automotive telematics and vehicle tracking device designed for:

- OBD vehicle data acquisition

- CAN communication

- LTE communication

- GNSS/GPS tracking

- ESP32 WiFi/Bluetooth communication

- USB programming and debugging

This document defines the functional tests required after PCB assembly to validate proper hardware operation.

# Test Equipment Required

| Equipment | Purpose |
| --- | --- |
| Bench Power Supply (12V, 3A minimum) | PCB power input |
| USB Type-C cable | ESP32 programming/testing |
| CP2102 USB-UART | UART communication |
| LTE SIM card | GSM/LTE testing |
| LTE antenna | Cellular communication |
| GNSS antenna | GPS testing |
| Multimeter | Voltage verification |
| Oscilloscope (optional) | Signal verification |
| CAN analyzer (optional) | CAN bus validation |

# Power-On Inspection

## Visual Inspection

Verify:

- No solder bridges

- Correct polarity of capacitors/diodes

- GSM module soldering quality

- ESP32 module alignment

- U.FL connectors mounted correctly

- SIM holder solder joints

- No missing components

# Input Power Test

## Test Condition

Apply:

- 12V DC input

## Expected Result

| Test Point | Expected Voltage |
| --- | --- |
| VIN | 12V |
| +5V Rail | 5V ±5% |
| +3V3 Rail | 3.3V ±5% |
| GSM_VBAT | 3.8V – 4.2V |

## Pass Criteria

- No overheating

- No excessive current draw

- Stable power rails

# ESP32 Functional Test

## USB Detection Test

### Procedure

- Connect USB cable

- Connect to PC

- Verify COM port detection

### Expected Result

- CP2102 enumerates correctly

- COM port visible on PC

## ESP32 Programming Test

### Procedure

- Flash test firmware

- Open serial monitor

### Expected Result

- Firmware uploads successfully

- Boot log visible at UART

## ESP32 GPIO Test

### Verify:

- Status LED operation

- UART communication

- Reset functionality

# GSM/LTE Functional Test

## SIM Detection Test

### Procedure

- Insert active SIM card

- Power ON board

- Send AT commands

### Expected Result

AT response:

AT
OK

SIM status:

AT+CPIN?
+CPIN: READY

## Network Registration Test

### Command

AT+CREG?

### Expected Result

+CREG: 0,1
or
+CREG: 0,5

## Signal Quality Test

### Command

AT+CSQ

### Expected Result

RSSI greater than:

10

Recommended:

15+

## LTE Data Test

### Verify

- Internet connectivity

- MQTT/HTTP communication

- Stable LTE operation

# GNSS/GPS Functional Test

## GNSS Antenna Test

### Procedure

- Connect GNSS antenna

- Place device near open sky

- Enable GNSS

### Expected Result

- Satellite lock achieved

- Latitude/Longitude received

## GNSS Data Verification

### Verify

- Valid NMEA data

- Stable location updates

- Multiple satellites detected

# CAN Bus Functional Test

## CAN Communication Test

### Procedure

- Connect to vehicle CAN bus

- Monitor CAN RX/TX

### Expected Result

- CAN frames received successfully

- No bus-off state

- Stable communication

# USB Interface Test

## Verify

- USB enumeration

- UART communication

- Stable data transfer

# Thermal Validation

## Run Duration

30 minutes continuous operation

## Verify

- No excessive heating

- Stable GSM operation

- No random resets

# Power Consumption Validation

## Verify

| Mode | Expected Behavior |
| --- | --- |
| Idle | Low current |
| LTE TX | Higher burst current |
| GPS Active | Stable operation |

# RF Verification

## LTE

Verify:

- Stable network connection

- Good RSSI

- No frequent disconnects

## GNSS

Verify:

- Stable GPS lock

- Satellite detection

- No intermittent signal loss

# Final Assembly Validation

## Verify

- All mounting holes aligned

- SIM insertion/removal works correctly

- U.FL connectors mechanically stable

- OBD connector aligned correctly

# Firmware Validation

## Verify

- Watchdog operation

- GSM recovery logic

- Brownout handling

- CAN data parsing

- GPS parsing

- Stable UART communication

# Pass / Fail Criteria

## PASS

Board considered PASS if:

- All power rails stable

- ESP32 programmable

- GSM registers on network

- GPS acquires location

- CAN communication functional

- No overheating

- Stable long-duration operation

## FAIL

Board considered FAIL if:

- Any short circuit present

- GSM instability observed

- GPS lock failure

- ESP32 programming failure

- Continuous reset/reboot issue

- Excessive temperature rise

# Notes for JLCPCB Assembly Team

## Important Components

Critical assembly inspection required for:

- A7672 GSM module

- ESP32-WROOM module

- U.FL RF connectors

- SIM holder

- CP2102 USB-UART IC

- TPS62160 power regulator

- CAN transceiver

## RF Precautions

- Avoid damaging RF connectors

- Ensure proper grounding around RF section

- Maintain solder quality on GSM module pads

## Power Section Precautions

- Verify polarity of all capacitors

- Ensure proper soldering of high-current GSM power path

- Inspect regulator thermal pads carefully

# Revision

Document Version: V1.0

PCB Type:

ESP32 + A7672 OBD Vehicle Tracking PCB

Prepared For:

JLCPCB SMT Assembly and Functional Validation