# GPS Asset Tracker

Using Arduino UNO, NEO-6M GPS, SIM800L GSM, DS3231 RTC & SD Card Module

# Certificate

This project report entitled 'GPS Asset Tracker' has been successfully completed as a part of Embedded Systems training.

# Abstract

A GPS Asset Tracker monitors the real-time location of assets using GPS, sends updates using GSM, records time with RTC, and stores data on an SD card.

# Introduction

This project demonstrates a simple GPS-based asset tracking system using Arduino UNO.

# Objectives

• Track GPS location
• Send SMS alerts
• Store data on SD card
• Record date & time

# Components

Arduino UNO
NEO-6M GPS
SIM800L GSM
DS3231 RTC
SD Card Module
Breadboard
Jumper Wires

# Block Diagram

GPS → Arduino UNO ← RTC
Arduino UNO → GSM
Arduino UNO → SD Card

# Circuit Connections

GPS TX→D4 RX→D3
SIM800L TX→D8 RX→D7
RTC SDA→A4 SCL→A5
SD CS→D10 MOSI→D11 MISO→D12 SCK→D13

# Working Principle

Arduino reads GPS coordinates, adds RTC timestamp, saves data to SD card and sends location through GSM.

# Flowchart

Start → Initialize → Read GPS → Read RTC → Save SD → Send SMS → Repeat

# Arduino Code

#include <TinyGPS++.h>
#include <SoftwareSerial.h>
TinyGPSPlus gps;
SoftwareSerial gpsSerial(4,3);
void setup(){
 Serial.begin(9600);
 gpsSerial.begin(9600);
}
void loop(){
 while(gpsSerial.available()) gps.encode(gpsSerial.read());
 if(gps.location.isUpdated()){
  Serial.println(gps.location.lat(),6);
  Serial.println(gps.location.lng(),6);
 }
}

# Applications

Vehicle Tracking
Fleet Management
School Bus Tracking
Asset Security

# Advantages

Real-time monitoring
Low cost
Easy to implement

# Conclusion

The project provides an economical solution for location monitoring using Arduino.

# Viva Questions

1. What is GPS?
2. Why use GSM?
3. What is RTC?
4. Why use SD Card?

# References

Arduino Documentation
TinyGPS++ Library
NEO-6M Datasheet